Technical Analysis: Gateway, Transceiver, and Antenna Selection for Smart Road Reflector Network

Document Reference: TA-2026-005
Project: SmartReflect Platform Integration
Subject: LoRaWAN Gateway, Node Transceiver, and Compact Antenna Selection for Smart Road Infrastructure

1. Problem Statement

The SmartReflect platform requires a reliable wireless communication architecture capable of supporting low-power road-embedded sensor nodes and bidirectional communication with a cloud-based backend. The system must transmit sensor data from each smart reflector node to the cloud and receive configuration or control commands from the cloud back to the node.

The primary engineering challenge is to select the best combination of:

- LoRaWAN gateway

- Node transceiver or LoRaWAN module

- Compact antenna

- Frequency band

- Device class

- Cloud network architecture

- RF layout strategy

- Power management strategy

The selected communication architecture must balance:

- Cost

- Range

- Power consumption

- Bidirectional communication capability

- Physical size

- Antenna integration constraints

- Low-profile housing limitations

- Ohio road deployment conditions

- Gateway availability

- Transceiver availability

- RF performance near ground level

- Scalability to multiple nodes

- Firmware complexity

- Certification and regulatory risk

- Long-term productization potential

The wireless system cannot be selected only by maximum theoretical range. In this application, the node will be installed inside a compact smart road reflector housing close to pavement level, where RF propagation is degraded by ground proximity, enclosure material, battery placement, internal electronics, and antenna size limitations.

2. System Communication Objective

The SmartReflect system is expected to operate as a distributed roadway sensing network.

The intended communication flow is:

Smart reflector node

↓

LoRaWAN uplink

↓

LoRaWAN gateway

↓

Cloud network server

↓

Application backend / analytics

↓

Optional downlink command

↓

Gateway

↓

Smart reflector node

The node must support two communication directions:

- Uplink: Sensor data, event summaries, battery voltage, temperature, vibration features, and diagnostic information.

- Downlink: Configuration changes, reporting interval updates, wake-up threshold changes, firmware mode changes, calibration parameters, and possible control commands.

Because the node is battery-powered and solar-assisted, downlink communication must be carefully managed to avoid excessive energy consumption.

3. Key Engineering Questions

The communication architecture must answer the following questions:

Which gateway should be used for laboratory and early field testing?

Which transceiver or LoRaWAN module should be used in the node?

Should the node use a complete LoRaWAN module or a raw SX1262 transceiver?

Which antenna type can fit inside a 125 × 125 × 25 mm road reflector housing?

How should bidirectional communication be implemented without draining the battery?

Should the node operate as LoRaWAN Class A or Class C?

How much range is realistic from a pavement-level node?

How should the antenna be placed inside the housing?

How should the PCB be designed to avoid RF detuning?

What should be selected for prototype versus final product?

4. Communication Architecture Requirements

4.1 Functional Requirements

The communication subsystem must support:

- US915 LoRaWAN operation

- Uplink sensor messages

- Downlink configuration messages

- Cloud-based processing

- Gateway packet forwarding

- Low-power node operation

- Compact antenna integration

- Multi-node scalability

- Outdoor field testing

- Local laboratory testing

- Reliable RSSI/SNR logging

- Secure device authentication

4.2 Electrical Requirements

The node radio subsystem must be compatible with:

Battery: LiFePO₄ 18650

Nominal battery voltage: 3.2 V

System rail: regulated 3.3 V

MCU: ESP32-C3 or equivalent

Power architecture: buck-boost regulated 3.3 V

Sleep-first operating strategy

The radio must tolerate:

- Short high-current transmit bursts

- Long sleep intervals

- Wake-up from deep sleep

- Downlink receive windows

- Temperature variation

- Low-voltage battery operation

4.3 Mechanical Requirements

The node antenna must fit inside a low-profile reflector housing:

Target housing footprint: approximately 125 mm × 125 mm

Maximum housing height: approximately 25 mm

Antenna must remain inside RF-transparent housing region

Antenna must be separated from battery and metal components

Antenna must survive vibration, shock, heat, and moisture

4.4 Environmental Requirements

The RF system must operate under:

- Cold Ohio winters

- Hot summer pavement conditions

- Rain and humidity

- Road salt exposure

- Internal condensation risk

- Mechanical vibration

- Low antenna height above pavement

- Possible partial obstruction by snow, dirt, or debris

5. LoRaWAN Device Class Analysis

5.1 Class A

Class A is the lowest-power LoRaWAN operating mode and should be considered the default operating mode for the SmartReflect node.

Operation:

Node sleeps

↓

Node wakes up

↓

Node transmits uplink

↓

Node opens RX1 receive window

↓

Node opens RX2 receive window

↓

Node returns to sleep

Advantages

- Lowest energy consumption

- Best fit for solar/battery-powered nodes

- Suitable for event-based reporting

- Supports bidirectional communication

- Gateway can send downlink after node uplink

- Best fit for compact LiFePO₄-powered reflector node

Disadvantages

- Downlink cannot arrive at any arbitrary time

- Cloud command must wait until the node transmits

- Not suitable for immediate real-time actuation

- Requires firmware to stay awake long enough after uplink for RX1/RX2 windows

Assessment for SmartReflect

Class A should be the primary operating mode for the smart road reflector.

Most downlink commands in this project are expected to be configuration commands, not emergency real-time actuation commands. Examples:

Change sampling interval

Change acceleration threshold

Change piezo wake-up threshold

Change reporting schedule

Request diagnostic mode

Change LoRa data rate policy

Request firmware state report

These commands can wait until the node performs its next uplink.

Recommended status:

Primary node mode: LoRaWAN Class A

5.2 Class C

Class C keeps the receiver open most of the time, allowing much lower-latency downlinks.

Operation:

Node transmits uplink

↓

Node keeps receive window open

↓

Cloud can send downlink almost anytime

Advantages

- Low downlink latency

- Cloud can send commands quickly

- Useful for powered devices

- Useful for always-listening infrastructure devices

Disadvantages

- High energy consumption

- Receiver remains active for long periods

- Not suitable for small solar/battery-powered road reflector nodes

- Requires larger energy budget

- May reduce battery life significantly

Assessment for SmartReflect

Class C should not be the normal operating mode for the road reflector node. It may be useful only in special cases:

Bench testing

Gateway/network validation

Temporary diagnostic mode

Externally powered prototypes

Firmware update support

Controlled short-duration field test

Recommended status:

Normal operation: Avoid Class C

Diagnostic/testing mode: Optional

5.3 Recommended Device Class Strategy

The recommended strategy is dual-mode firmware:

Production/default mode:

Class A

Temporary diagnostic mode:

Class C only when explicitly enabled and energy is available

Recommended behavior:

If battery is healthy and debug mode is active:

Allow temporary Class C listening

If battery is low:

Force Class A

If solar input is weak:

Force Class A

If enclosure temperature is too high:

Force Class A or disable radio temporarily

6. Gateway Selection Analysis

6.1 Gateway Requirements

The gateway must support:

- US915 LoRaWAN operation

- 8-channel packet forwarding

- Ethernet backhaul

- Wi-Fi setup or backhaul

- Optional LTE backhaul

- Cloud network server integration

- The Things Stack / TTN compatibility

- ChirpStack compatibility

- Downlink support

- Indoor lab use

- Possible field test use with protected installation

- External antenna connector

- Stable operation for multiple nodes

6.2 Candidate Gateway: RAK Wireless WisGate Edge Lite 2 RAK7268C US915 LTE

The selected gateway candidate is:

RAK Wireless WisGate Edge Lite 2

Model family: RAK7268 / RAK7268C

Region: US915

Optional backhaul: LTE

6.3 Why This Gateway Fits the Project

The RAK7268C is a strong gateway choice for the prototype phase because it provides a balance between cost, capability, and ease of integration.

Main advantages:

- Full 8-channel LoRaWAN gateway

- Supports US915 operation

- Compatible with cloud packet forwarding

- Supports Ethernet connectivity

- Supports Wi-Fi setup or client mode

- Optional LTE backhaul

- Compact indoor form factor

- Web-based configuration

- Good fit for lab testing

- Compatible with common LoRaWAN network servers

- Supports uplink and downlink routing

- Strong ecosystem compatibility with RAK node modules

6.4 Gateway Role in the Architecture

The gateway should not perform heavy application processing during the first prototype phase.

Recommended role:

RF packet reception

LoRaWAN packet forwarding

Cloud backhaul

Downlink packet transmission

Basic gateway diagnostics

Processing should remain in the cloud/backend.

This separation keeps the gateway simple and makes the system scalable.

6.5 Gateway Limitations

The RAK7268C is an indoor gateway. It is appropriate for laboratory development and controlled field tests, but it is not ideal as a permanently exposed outdoor roadside unit unless installed inside a protected enclosure.

Limitations:

- Indoor enclosure rating

- Needs protected mounting location

- Requires power supply

- Gateway antenna placement strongly affects range

- Ground-level nodes require elevated gateway antenna placement

- LTE version increases cost and power requirements

- Not equivalent to a rugged outdoor highway gateway

6.6 Recommended Gateway Usage

For prototype and lab testing:

Use RAK7268C indoors or in a protected location

Connect to TTN or ChirpStack

Use Ethernet when possible

Use LTE only when needed for field tests

Use external antenna connector for range experiments

For later field deployment:

Evaluate outdoor IP-rated gateway

Mount gateway antenna elevated

Use PoE or solar-backed infrastructure power

Use LTE backhaul only where wired internet is unavailable

6.7 Gateway Recommendation

Recommended gateway for current phase:

RAK Wireless WisGate Edge Lite 2 RAK7268C US915 LTE

Reason:

Best balance of cost, LoRaWAN capability, cloud connectivity, bidirectional support, and development convenience.

7. Transceiver / Node Radio Selection Analysis

The node radio decision has two main paths:

- Use a complete LoRaWAN module with integrated stack.

- Use a raw transceiver chip and implement the LoRaWAN stack in firmware.

7.1 Option A: RAK3172 LoRaWAN Module

Description

The RAK3172 is a LoRaWAN module based on the STM32WLE5 wireless MCU. It includes the LoRa radio and an MCU capable of running the LoRaWAN stack.

Typical integration method:

ESP32-C3

↓ UART

RAK3172

↓ RF output

Antenna

Advantages

- Strong ecosystem match with RAK gateway

- Supports US915

- Supports LoRaWAN Class A/B/C

- UART AT command interface

- Reduces firmware complexity

- Integrated LoRaWAN stack

- Good for fast prototyping

- Low sleep current

- Reduces risk of RF stack implementation errors

- Good documentation and development ecosystem

- Useful for bidirectional LoRaWAN testing

Disadvantages

- Higher module cost than raw transceiver chip

- Requires communication between ESP32-C3 and module

- May duplicate MCU resources because the module already has an MCU

- Less control than raw SX1262

- Firmware behavior depends on module AT command implementation

- Product cost may be higher than chip-level design

Assessment

The RAK3172 is the best first choice for the prototype because it minimizes software and RF stack risk. It allows the project to focus on sensor behavior, power architecture, housing, antenna placement, and cloud logic.

Recommended status:

Prototype: Strongly recommended

Field prototype: Recommended

Final product: Possible, if cost is acceptable

7.2 Option B: Seeed Wio-E5 Module

Description

The Wio-E5 is another LoRaWAN module based on the STM32WLE5 architecture. It also provides an AT command interface and supports LoRaWAN operation.

Typical integration method:

ESP32-C3

↓ UART

Wio-E5

↓ RF output

Antenna

Advantages

- Compact module

- Low sleep current

- AT command interface

- Good for breadboard and bench testing

- Supports US915

- Useful alternative supplier/module

- Good for fast prototyping

- Based on similar STM32WLE5/SX126x architecture

Disadvantages

- Not the same vendor ecosystem as selected gateway

- Different AT command behavior from RAK modules

- Requires firmware adaptation if switching from RAK3172

- Module variant and certification status must be verified

- May not be preferred if the project standardizes around RAK hardware

Assessment

The Wio-E5 is a good secondary module for comparison or backup. It is valuable as an alternate module, but the RAK3172 is more consistent with the selected RAK gateway ecosystem.

Recommended status:

Prototype backup: Recommended

Second-source evaluation: Recommended

Primary selection: Secondary to RAK3172

7.3 Option C: Semtech SX1262 Raw Transceiver

Description

The SX1262 is a raw LoRa transceiver chip. It provides the RF modem but does not independently provide a complete application-level LoRaWAN device unless the host MCU implements or integrates a LoRaWAN stack.

Typical integration method:

ESP32-C3

↓ SPI

SX1262

↓ RF matching network

Antenna

Advantages

- Excellent RF performance

- High link budget

- High output power capability

- Lower chip-level BOM cost at scale

- Smaller final PCB if designed correctly

- More control over radio firmware

- Better for optimized product design

- Avoids duplicate module MCU

- Can reduce final product cost

Disadvantages

- Higher firmware complexity

- Requires LoRaWAN stack integration

- Requires RF layout expertise

- Requires impedance matching network

- Requires RF validation

- Requires certification effort

- More risk during early prototype

- Longer development time

- SPI integration and radio interrupt handling required

Assessment

The SX1262 is the best long-term custom PCB direction, but it is not the best first prototype path. It should be evaluated after the sensor, housing, battery, antenna placement, and communication architecture are validated with a module.

Recommended status:

First prototype: Not preferred

RF benchmark board: Recommended

Final custom PCB: Strong candidate

8. Transceiver Comparison Matrix

| Parameter | RAK3172 | Wio-E5 | SX1262 Raw Chip |
| --- | --- | --- | --- |
| Type | LoRaWAN module | LoRaWAN module | Raw transceiver |
| Host interface | UART AT / RUI API options | UART AT | SPI |
| LoRaWAN stack | Integrated | Integrated | Host MCU required |
| Firmware complexity | Low | Low/medium | High |
| Prototype speed | High | High | Low |
| Final BOM cost | Medium | Medium | Low at scale |
| RF layout difficulty | Medium | Medium | High |
| Sleep power | Very low | Very low | Very low chip-level |
| Bidirectional support | Good | Good | Depends on stack |
| Class A support | Yes | Yes | Depends on stack |
| Class C support | Yes | Yes | Depends on stack |
| US915 support | Yes | Yes | Yes with correct design |
| Certification complexity | Lower | Lower | Higher |
| Best use | Main prototype | Backup module | Final custom PCB |

9. Recommended Transceiver Strategy

The recommended development path is staged:

Phase 1:

Use RAK3172 module with the RAK7268C gateway.

Phase 2:

Keep Wio-E5 as an alternate module for comparison.

Phase 3:

Evaluate SX1262 raw transceiver for final custom PCB.

Recommended immediate selection:

Gateway:

RAK7268C US915 LTE

Node module:

RAK3172-T or RAK3172 US915 variant with IPEX/U.FL option if external antenna is needed

Antenna:

Flexible FPC antenna with U.FL connector for first compact housing prototype

For Ohio road conditions, prefer the temperature-rated RAK3172 variant if available and compatible with the required certification and antenna interface.

10. Antenna Selection Analysis

10.1 Antenna Requirements

The antenna is one of the most critical design decisions because the node will be placed close to the ground inside a compact enclosure.

The antenna must:

- Fit inside 125 × 125 × 25 mm housing

- Operate in US915 band

- Be mechanically stable

- Survive vibration and thermal cycling

- Avoid detuning from battery, PCB, metal, and pavement

- Provide acceptable range at pavement level

- Be easy to assemble in prototype units

- Support repeatable RF performance

- Remain inside an RF-transparent housing region

10.2 Main Antenna Options

The viable compact antenna options are:

- Flexible FPC antenna

- Ceramic chip antenna

- Helical spring antenna

- External gateway antenna upgrade

The gateway antenna is separate from the node antenna but should be evaluated as part of the full link.

11. Option 1: Flexible FPC Antenna

11.1 Description

A flexible FPC antenna is a thin adhesive antenna connected to the PCB through a coaxial cable and U.FL/IPEX connector.

Typical placement:

Inside upper polymer wall

Away from battery

Away from metal

Away from large copper areas

Near RF-transparent housing region

11.2 Advantages

- Best fit for compact reflector housing

- Can be placed away from the main PCB ground plane

- Can be attached to inner plastic wall

- Better RF performance than many chip antennas in constrained layouts

- Low height

- Flexible placement

- Good for prototypes

- Avoids strict chip antenna PCB layout constraints

- Allows antenna position experiments

- Easy to replace during testing

11.3 Disadvantages

- Requires U.FL/IPEX connector

- Requires coax cable routing

- Adds assembly step

- Adhesive bond must survive heat and moisture

- Cable must be strain-relieved

- Antenna performance depends on housing material and placement

- Needs RF validation inside final housing

11.4 Assessment

The FPC antenna is the best first prototype antenna for the SmartReflect node. It provides the best balance of range, compact packaging, and placement flexibility.

Recommended status:

Prototype: Strongly recommended

Field prototype: Recommended

Final product: Strong candidate

12. Option 2: Ceramic Chip Antenna

12.1 Description

A ceramic chip antenna is a small SMD antenna mounted directly on the PCB. It minimizes physical size but requires careful PCB layout.

12.2 Advantages

- Smallest physical antenna option

- Low profile

- No cable

- No manual antenna placement

- Good for automated manufacturing

- Mechanically robust once soldered

- Good for high-volume production if RF layout is validated

12.3 Disadvantages

- Highly sensitive to PCB ground plane design

- Requires strict keep-out area

- Can be detuned by battery and enclosure

- Lower efficiency in constrained housing

- Harder to tune after assembly

- RF range may be reduced near ground level

- Requires VNA tuning and possibly matching network optimization

12.4 Assessment

A ceramic chip antenna is attractive for a future miniaturized production version, but it is not the best first antenna for the prototype because the housing and RF environment are still uncertain.

Recommended status:

Prototype: Not preferred

RF miniaturization study: Recommended

Final compact product: Possible after tuning

13. Option 3: Helical Spring Antenna

13.1 Description

A helical spring antenna is a compact 915 MHz coil antenna soldered directly to the PCB or mounted through a small pad.

13.2 Advantages

- Low cost

- Simple PCB interface

- Better tolerance than some chip antennas

- No coax connector

- Reasonable RF performance

- Easy to test on prototypes

- Can fit if vertical or horizontal clearance is available

13.3 Disadvantages

- Requires physical volume

- May be too tall for a 25 mm housing if vertical

- Can deform under mechanical shock

- Must be protected from compression

- Detunes if bent or near metal

- Less repeatable than FPC in rugged road environment

- May create mechanical reliability concerns

13.4 Assessment

A helical spring antenna is useful for early bench testing and low-cost experiments, but the mechanical risk inside a road reflector is higher than with an FPC antenna.

Recommended status:

Bench testing: Acceptable

Road reflector prototype: Secondary

Final product: Not preferred unless mechanically protected

14. Option 4: Gateway External Antenna Upgrade

14.1 Description

The gateway antenna can be upgraded independently of the node antenna. Raising the gateway antenna and using an appropriate higher-gain external antenna can improve link reliability.

14.2 Advantages

- Improves network coverage without increasing node power

- Does not increase node size

- Does not increase node energy consumption

- Helps compensate for low antenna height of road nodes

- Useful for field testing

14.3 Disadvantages

- Higher-gain antennas must remain compliant with regional EIRP limits

- Outdoor mounting requires lightning and surge protection

- Cable loss can reduce benefit if coax run is long

- Not a substitute for good node antenna design

- Directional or high-gain antenna patterns may create coverage holes

14.4 Assessment

For field tests, the gateway antenna should be elevated and optimized before increasing node transmit power. It is usually better to improve gateway placement and antenna installation than to increase node energy consumption.

Recommended status:

Field test: Recommended

Permanent deployment: Recommended with compliance review

15. Antenna Comparison Matrix

| Parameter | FPC Antenna | Ceramic Chip Antenna | Helical Spring Antenna |
| --- | --- | --- | --- |
| Space requirement | Low height, medium area | Very small | Medium 3D volume |
| PCB difficulty | Low/medium | High | Low |
| Assembly difficulty | Medium | Low after PCB assembly | Low/medium |
| RF performance in compact housing | Good | Variable | Medium |
| Sensitivity to ground plane | Medium | High | Medium |
| Mechanical durability | Good if bonded | Excellent | Medium/low |
| Prototype flexibility | Excellent | Poor | Medium |
| Final manufacturability | Medium | Excellent | Medium |
| Best use | First prototype | Future miniaturized product | Bench/low-cost test |

16. Recommended Antenna Strategy

Recommended node antenna for first prototype:

Flexible FPC 902–928 MHz / 915 MHz antenna

Connection: U.FL / IPEX

Placement: inside upper RF-transparent side wall

Recommended alternate antennas to evaluate:

Taoglas FXP290 or similar 915 MHz FPC antenna

Molex 868/915 MHz FPC antenna

Johanson 0915AT43A0026 ceramic chip antenna for miniaturization study

Generic 915 MHz helical spring antenna for bench comparison

Recommended PCB provision:

Add U.FL connector footprint

Add π-matching network footprint

Add optional chip antenna footprint only if PCB space allows

Add RF test point or RF switch footprint if possible

Keep antenna feed short

Maintain 50 Ω controlled impedance trace

Provide antenna keep-out area

17. RF Placement in the Smart Reflector Housing

17.1 Recommended Antenna Location

The antenna should be placed in the RF zone of the housing:

Upper or side internal wall

Behind RF-transparent polymer

Away from LiFePO₄ battery

Away from metal mass

Away from screw bosses if metallic

Away from large PCB copper pours

Away from piezo cantilever mass

Above or laterally offset from main PCB

17.2 Why Not Place the Antenna Near the Battery

The 18650 battery is a large conductive cylindrical object. If the antenna is too close to the cell, it may detune the antenna and reduce radiation efficiency.

Avoid:

Antenna directly beside battery

Antenna under battery

Antenna crossing battery wires

Antenna near steel battery clips

Antenna near large copper planes

17.3 Why the Housing Material Matters

The antenna should radiate through a polymer section of the housing. Avoid metal or carbon-loaded materials near the antenna.

Preferred antenna window materials:

Polycarbonate

PC/ABS

ABS

Other RF-transparent polymer

Avoid near antenna:

Metal inserts

Steel screws

Aluminum shield walls

Carbon-filled plastic

Large copper pours

Battery can

17.4 Recommended RF Zone

The RF zone should be physically defined in CAD and PCB layout.

Recommended RF zone rule:

No battery

No metal mass

No large copper plane

No power inductor

No charger heat source

No piezo tip mass

No structural metal insert

18. Expected Range Analysis

18.1 Theoretical Range

The SX1262-class radio can achieve long range under line-of-sight conditions, especially when paired with a well-placed gateway antenna. However, theoretical range does not represent the smart reflector environment.

18.2 Realistic Road-Level Range

For a node inside a compact road reflector, realistic range is limited by:

- Ground proximity

- Low antenna height

- Pavement absorption

- Vehicle obstruction

- Internal battery shielding

- Enclosure loss

- Small antenna efficiency

- Gateway antenna height

- Urban clutter

- Weather and snow coverage

Expected practical ranges should be validated experimentally.

Preliminary expectation:

Laboratory indoor testing:

Tens to hundreds of meters depending on walls and antenna placement

Outdoor line-of-sight with elevated gateway:

Several kilometers possible

Road-level reflector node with compact internal antenna:

Hundreds of meters is a more realistic initial design target

For the first field test, the project should not assume 10+ km range from a ground-level reflector node. The gateway antenna must be elevated and the node antenna must be carefully placed.

19. Cloud and Network Server Strategy

19.1 Recommended Prototype Network

For early development:

Network server:

The Things Stack / TTN or ChirpStack

Gateway:

RAK7268C as packet forwarder

Application backend:

MQTT, webhook, Node-RED, FastAPI, AWS, or similar

19.2 Why Use Cloud Processing

Cloud processing allows:

- Centralized analytics

- Easier model updates

- Fleet monitoring

- Gateway-independent logic

- Long-term data storage

- Remote configuration

- Scalable dashboards

- Integration with maps or road-condition systems

19.3 Downlink Payload Strategy

Downlinks should be compact byte-level payloads, not JSON strings.

Recommended downlink format:

Byte 0: command ID

Byte 1: parameter ID

Byte 2–N: compact value

Example commands:

0x01: Set reporting interval

0x02: Set accelerometer threshold

0x03: Enable diagnostic mode

0x04: Request battery report

0x05: Change sampling window

0x06: Change LoRa data rate policy

0x07: Enter temporary Class C mode

Payloads should typically remain very short to reduce airtime, energy use, and downlink congestion.

20. Power Management Considerations

20.1 Radio Power Strategy

The radio must be off or asleep most of the time.

Recommended operation:

ESP32-C3 deep sleep

↓

Piezo wake-up event

↓

ESP32 enables accelerometer and radio

↓

Node transmits uplink

↓

Node waits for RX1/RX2 downlink windows

↓

Node processes optional command

↓

Radio returns to sleep

20.2 Avoiding Excessive Downlink Energy

Downlinks require the node to keep the receiver active at scheduled times. Frequent downlinks increase energy consumption and reduce battery life.

Recommended policy:

Use uplink-driven downlinks

Avoid frequent confirmed messages

Avoid Class C except during debug

Batch configuration commands

Use short payloads

Let the node sleep aggressively

20.3 Transmit Power Policy

Do not always transmit at maximum power unless needed.

Recommended policy:

Start with conservative transmit power

Measure RSSI/SNR

Use ADR where appropriate

Increase power only if link margin is poor

Avoid maximum TX power during normal operation unless field data justifies it

For a road-level node, antenna placement and gateway elevation will usually matter more than small changes in transmit power.

21. Regulatory and Certification Considerations

21.1 US915 Operation

The system should use US915 regional settings for operation in the United States.

Required checks:

Correct frequency plan

Correct sub-band configuration

Correct gateway channel plan

Correct node channel mask

FCC-compliant module or certified radio path

Antenna gain compliance

EIRP compliance

Duty cycle / dwell time compliance where applicable

21.2 Module vs. Raw Chip Certification

Using a certified LoRaWAN module can reduce certification risk.

Comparison:

Certified module:

Lower certification burden

Faster prototype-to-product path

Less RF design risk

Raw SX1262:

More design freedom

Lower cost at scale

Higher RF/certification burden

Requires full RF validation

For early prototypes, certified modules are preferable.

22. Supply Chain and Export / Import Cost Risk

The communication subsystem should be evaluated for procurement risk.

Factors to review:

Distributor availability

Lead time

Module lifecycle status

Certification variant availability

US915 variant availability

Antenna connector variant availability

Tariff/import cost

Country of origin

Minimum order quantity

Second-source alternatives

Recommended purchasing strategy:

Prototype:

Buy from authorized distributors or manufacturer store

Field prototype:

Standardize one main module and one backup module

Product:

Request formal production quotes and confirm tariff/import exposure before design freeze

Do not freeze the PCB until the exact radio module part number, antenna connector option, certification variant, and frequency variant are confirmed.

23. Recommended Prototype Architecture

The recommended first prototype configuration is:

Gateway:

RAK WisGate Edge Lite 2 RAK7268C US915 LTE

Network server:

The Things Stack / TTN for lab

ChirpStack or commercial TTI/AWS option for scaling review

Node transceiver:

RAK3172 US915 variant with IPEX/U.FL antenna interface

Node MCU:

ESP32-C3

Antenna:

915 MHz flexible FPC antenna with U.FL/IPEX connector

Operating mode:

LoRaWAN Class A by default

Class C only for temporary diagnostics

RF placement:

Antenna in upper/side RF-transparent housing zone

Battery on opposite side

PCB with 50 Ω RF feed and π-matching footprint

24. Recommended Final Product Direction

The recommended long-term direction is:

Short term:

RAK3172 module for working prototype

Medium term:

Compare RAK3172 against Wio-E5 and SX1262 evaluation board

Long term:

Decide between certified module and raw SX1262 based on volume, cost, certification path, and engineering resources

Decision rule:

If development speed and certification simplicity are more important:

Use RAK3172 module

If unit cost at scale becomes the dominant constraint:

Evaluate raw SX1262 custom design

If RAK supply or pricing becomes problematic:

Evaluate Wio-E5 as alternate module

25. Test Plan

25.1 Gateway Test

Test:

Gateway registration

US915 frequency plan

Ethernet backhaul

Wi-Fi backhaul

LTE backhaul if available

Packet forwarder mode

Cloud uplink reception

Cloud downlink scheduling

Gateway RSSI/SNR reporting

Deliverables:

Gateway configuration report

Network server integration report

Uplink/downlink test log

25.2 Transceiver Test

Test:

RAK3172 Class A uplink

RAK3172 downlink RX1/RX2

Temporary Class C behavior

Sleep current

TX current

RX current

Join time

Rejoin behavior

Payload encoding

Failure recovery

Deliverables:

Radio power profile

LoRaWAN command interface document

Downlink payload format

Firmware state machine

25.3 Antenna Test

Test:

FPC antenna mounted in different housing locations

Antenna near battery versus away from battery

Antenna under polymer wall

Antenna near metal screw bosses

Housing open versus closed

Ground-level performance

Gateway antenna elevation changes

RSSI/SNR at multiple distances

Deliverables:

Antenna placement recommendation

RF layout guideline

Range test report

Housing RF loss estimate

25.4 Field Test

Test:

Outdoor short-range test

Road-level placement test

Elevated gateway test

Battery-powered node test

Solar charging active

Event-driven uplinks

Downlink configuration updates

RSSI/SNR over time

Temperature impact on radio behavior

Deliverables:

Field communication report

Recommended transmit power

Recommended data rate

Recommended antenna placement

Final gateway placement guideline

26. Risk Register

| Risk | Severity | Likelihood | Mitigation |
| --- | --- | --- | --- |
| Poor range from ground-level node | High | High | Use FPC antenna, elevate gateway antenna, optimize RF zone |
| Antenna detuning from battery | High | Medium/high | Place antenna away from 18650 and metal |
| Excessive downlink energy | Medium/high | Medium | Use Class A, short downlinks, avoid frequent Class C |
| Firmware complexity with raw SX1262 | High | Medium | Use RAK3172 for prototype |
| Gateway not suitable for outdoor deployment | Medium | Medium | Use protected install or outdoor gateway later |
| Wrong US915 sub-band configuration | High | Medium | Validate channel plan in gateway and node |
| RF loss due to housing material | Medium/high | Medium | Use RF-transparent polymer and test closed housing |
| Certification delay | High | Medium | Use certified module during prototype/product bridge |
| Import/tariff cost variation | Medium | Medium | Verify final quote and supplier country of origin |
| U.FL cable/connector failure | Medium | Medium | Strain relief, adhesive retention, vibration testing |
| Excessive transmit power consumption | Medium | Medium | Use link budget measurements and ADR strategy |

27. Final Recommendation

The recommended communication architecture for the current SmartReflect prototype is:

Gateway:

RAK Wireless WisGate Edge Lite 2 RAK7268C US915 LTE

Node radio:

RAK3172 US915 LoRaWAN module

Antenna:

915 MHz flexible FPC antenna with U.FL/IPEX connector

Operating mode:

LoRaWAN Class A by default

Cloud architecture:

Gateway as packet forwarder

Cloud-based processing

Short byte-level downlink commands

The RAK7268C provides the right development gateway capability for bidirectional LoRaWAN testing. The RAK3172 minimizes firmware and LoRaWAN stack complexity, which is valuable during the prototype stage. The flexible FPC antenna is the best first antenna option because it can be placed away from the battery and attached to an RF-transparent area of the housing.

The raw SX1262 should remain the preferred long-term custom PCB direction only after the project validates sensor detection, power budget, antenna placement, housing RF behavior, and cloud downlink control.

The most important RF design rule for the road reflector is:

Use a low-power LoRaWAN module first, place the antenna away from the battery and metal, operate the node in Class A, and validate real range from pavement level before optimizing for final cost.