Technical Analysis: Internal Component Placement for Smart Road Reflector Platform

Document Reference: TA-2026-004
Project: SmartReflect Platform Integration
Subject: Internal Component Placement, Thermal Zoning, Sensor Coupling, and Housing Architecture

1. Problem Statement

The SmartReflect platform requires an optimized internal component layout to support reliable operation inside a compact road-mounted reflector housing. The device must integrate sensing, power management, solar charging, wireless communication, and environmental protection while operating in harsh roadway conditions.

The main engineering challenge is to determine where each internal component should be placed so that the system achieves the best balance between:

- Mechanical durability

- Vibration transfer

- Sensor accuracy

- Thermal protection

- Battery safety

- RF performance

- Solar charging efficiency

- Water and salt resistance

- Manufacturability

- Cost

- Size constraints

- Serviceability

- Long-term reliability

The reflector housing is not only a mechanical enclosure. It functions as a combined structural, thermal, RF, sensing, and environmental protection subsystem.

The internal layout must therefore be designed as a system-level architecture, not as a simple packaging exercise.

2. System Context

The proposed SmartReflect node includes the following major subsystems:

- LiFePO₄ 18650 battery

- CN3058E LiFePO₄ solar charger

- 3.3 V buck-boost regulator

- ESP32-C3 microcontroller

- LoRa communication module

- LoRa antenna

- Piezoelectric wake-up sensor

- Accelerometer, such as BMA400 or equivalent

- Battery NTC thermistor

- Internal enclosure temperature sensor

- Power-stage temperature sensor

- Solar panel

- Protection circuitry

- Sealed road-rated housing

- Mechanical sensor mounting structure

The intended low-power operating sequence is:

System in deep sleep

↓

Piezoelectric element detects vibration

↓

Wake-up circuit triggers ESP32-C3

↓

ESP32-C3 enables accelerometer power

↓

Accelerometer captures short vibration window

↓

ESP32-C3 processes event features

↓

LoRa sends compact message

↓

System returns to deep sleep

This operating strategy allows the accelerometer, LoRa module, and other subsystems to remain off or in low-power states most of the time.

3. Placement Design Objectives

The internal placement must satisfy six major objectives.

3.1 Mechanical Objective

The device must survive tire loading, vibration, installation stress, pavement movement, and impact. Components must be mechanically secured and protected from shock.

3.2 Thermal Objective

The battery, charger, regulator, and electronics must remain within safe operating limits. The layout must prevent unsafe cold charging and summer overheating.

3.3 Sensor Objective

The accelerometer and piezoelectric element must be positioned to capture useful roadway vibration while minimizing false signals caused by loose internal structures.

3.4 RF Objective

The LoRa antenna must be placed where the housing material, battery, copper pours, and metal structures do not excessively attenuate or detune the RF signal.

3.5 Environmental Objective

The layout must reduce water ingress risk, corrosion exposure, condensation problems, and salt-related damage.

3.6 Manufacturing Objective

The design should be easy to assemble, test, seal, and eventually produce at scale.

4. Recommended Internal Zoning

The reflector should be divided into functional internal zones.

Top Layer:

Solar panel / optical cover / environmental cover

Upper Internal Layer:

Antenna zone

Internal temperature sensing

Solar input routing

Middle Internal Layer:

Battery zone

Power electronics zone

Main PCB

Lower Internal Layer:

Accelerometer rigid mount

Piezoelectric mechanical amplifier

Structural base coupled to pavement

Bottom Layer:

Adhesive / mechanical interface to pavement

Recommended zone layout:

Top View - Conceptual Layout

┌─────────────────────────────────────────────┐

│              Solar Panel Area           			    │

│                                             				│

│  Battery Zone                  RF Zone    			  │

│  LiFePO₄ 18650                 LoRa Antenna		 │

│  Battery NTC                   Polymer Wall		 │

│                                           				  │

│  Power Zone                    MCU Zone    		 │

│  CN3058E                       ESP32-C3    			 │

│  Buck-Boost                    LoRa Module  		│

│  Protection                    Debug Pads   		│

│                                            				 │

│  Sensor Zone                               			 │

│  Rigid Accelerometer + Piezo Cantilever     	│

└─────────────────────────────────────────────┘

Side view:

Side Section - Conceptual Layout

Solar panel / top protective window

┌─────────────────────────────────────┐

│ Upper cavity / RF-transparent area   │

│ Internal temperature sensor          │

│                                     │

│ LiFePO₄ 18650 + battery NTC          │

│                                     │

│ Main PCB: charger, regulator, MCU    │

│                                     │

│ Accelerometer rigidly mounted        │

│ Piezo cantilever + small mass        │

└─────────────────────────────────────┘

Structural base bonded to pavement

5. Battery Placement

5.1 Recommended Location

The LiFePO₄ 18650 battery should be placed in a dedicated battery zone, preferably along one side of the housing or in a central protected cavity depending on the available geometry.

Recommended placement:

Middle layer

Mechanically protected cavity

Away from direct solar panel heat

Away from antenna keep-out area

Close enough to charger to reduce wiring loss

Thermally monitored with direct NTC contact

5.2 Why the Battery Should Not Be Near the Top Surface

The top surface of the reflector may become hot under direct sun exposure. Placing the battery directly under the solar panel or top cover increases the risk of summer overheating.

Main risks:

- Higher battery temperature during summer

- Reduced battery lifetime

- Higher risk of charging cutoff due to high temperature

- Higher thermal stress

- Larger temperature swings

5.3 Why the Battery Should Not Be at the Extreme Bottom Without Isolation

The bottom of the housing is mechanically and thermally coupled to the pavement. In winter, this can expose the battery to cold pavement temperature. In summer, it can expose the battery to hot asphalt temperature.

A direct bottom placement can cause:

- Faster cooling during winter

- Higher heat transfer from pavement during summer

- Larger thermal cycling

- Mechanical shock transfer into the cell

5.4 Recommended Battery Mounting Strategy

The battery should be retained using a mechanically secure but electrically insulated holder.

Recommended features:

Rigid support structure

No loose movement

No direct compression on cell casing

Electrical insulation

Shock-tolerant retention

NTC attached directly to cell wall

Clearance from sharp edges

Protection from condensation paths

The battery should not be fully potted. Full potting may trap heat, make rework impossible, and place mechanical stress on the cell during thermal expansion.

5.5 Battery Thermal Sensor

A thermistor should be physically attached to the battery body.

Recommended location:

Midpoint on the side wall of the 18650 cell

Direct contact with the metal can

Secured with thermal adhesive or thermal tape

Electrically insulated from the cell body if required

Purpose:

- Prevent charging below 0 °C

- Prevent charging above high-temperature limit

- Detect dangerous internal temperature trends

- Log real battery thermal behavior during field tests

6. CN3058E Charger Placement

6.1 Recommended Location

The CN3058E charger should be placed on the main PCB near the battery zone but not directly underneath or directly touching the battery.

Recommended placement:

Power electronics zone

Near solar input path

Near battery connection

On PCB area with thermal copper

Thermally monitored if possible

Separated from accelerometer and RF antenna

6.2 Reasoning

The CN3058E is a linear charger. It dissipates heat according to input voltage, battery voltage, and charge current.

Approximate heat loss:

Power loss ≈ (VIN - VBAT) × ICHG

This heat can become significant in a sealed enclosure. The charger therefore needs:

- Copper area for heat spreading

- Thermal distance from the battery

- Temperature monitoring

- Firmware or hardware charge disable control

- Protection from summer overheating

6.3 Why It Should Not Be Used as the Primary Battery Heater

Although the charger produces heat, it should not be treated as a reliable heater for cold-weather operation.

Reasons:

- Charger heat only exists when solar energy is available

- Heat depends on charge current

- Charge must be disabled when the LiFePO₄ battery is below 0 °C

- Heat may remain localized at the PCB rather than warming the battery core

- Summer conditions can convert the same heat source into a thermal risk

The charger should be thermally managed, not intentionally relied upon for heating.

7. Buck-Boost Regulator Placement

7.1 Recommended Location

The 3.3 V buck-boost regulator should be placed close to the main load distribution point, near the ESP32-C3 and LoRa module, but thermally separated from the accelerometer and battery.

Recommended placement:

Main PCB power zone

Close to ESP32-C3 and LoRa supply path

Short high-current traces

Good local input/output capacitors

Separated from accelerometer

Separated from RF antenna

Thermal copper area available

7.2 Reasoning

The buck-boost regulator supplies the system from a LiFePO₄ cell whose voltage may be above or below 3.3 V during operation.

The regulator handles:

- ESP32-C3 current peaks

- LoRa transmit current peaks

- Sensor supply current

- Wake-up and active-mode transitions

Poor placement can cause:

- Voltage droop during LoRa transmission

- EMI coupling into the accelerometer

- RF interference

- Excessive thermal concentration

- Reduced regulator efficiency

7.3 Decoupling Requirements

The regulator should have:

Input capacitor close to regulator

Output capacitor close to regulator

Bulk capacitor near LoRa module

Local decoupling near ESP32-C3

Short ground return paths

Wide power traces

The LoRa module should also have a local energy reservoir, typically a larger bulk capacitor, to support transmission peaks.

8. ESP32-C3 Placement

8.1 Recommended Location

The ESP32-C3 should be located on the main PCB in the electronics zone, between the power system and the sensor/RF interfaces.

Recommended placement:

Main PCB central electronics zone

Close to accelerometer interface traces

Close to load-switch control

Close to LoRa UART/SPI interface

Not directly under solar panel hot spot

Not next to high-current charger trace

Accessible to debug/programming pads

8.2 Reasoning

The ESP32-C3 controls:

- System sleep/wake behavior

- Piezo wake event interpretation

- Accelerometer power enable

- Sensor sampling

- LoRa communication

- Battery voltage measurement

- Thermal protection logic

- Charge enable/disable behavior

The MCU should be close enough to all controlled subsystems to reduce trace length and noise, but not placed directly next to high-heat or high-current components.

8.3 Debug Access

For prototype units, the layout should include accessible:

UART programming pads

Boot/reset access

Ground test points

3.3 V test point

Battery voltage test point

Solar input test point

I²C/SPI test points

Wake signal test point

These can be removed or reduced in final product versions.

9. LoRa Module and Antenna Placement

9.1 Recommended Location

The LoRa module may be placed on the main PCB, but the antenna must be located near an RF-transparent region of the housing.

Recommended antenna placement:

Near side wall or upper corner

Behind polymer material

Away from battery

Away from metal screws

Away from large copper planes

Away from piezo mass

Away from structural metal inserts

9.2 Why the Antenna Should Be Away From the Battery

The 18650 cell is a large conductive cylindrical object. It can detune or shield the antenna, reducing LoRa range and reliability.

Avoid:

Antenna directly beside battery

Antenna under battery

Antenna surrounded by copper

Antenna behind metal frame

Antenna near steel fasteners

9.3 RF Keep-Out Zone

The antenna should have a defined keep-out area.

Recommended approach:

Use polymer wall or cover near antenna

Keep metal and battery away from antenna field

Avoid ground pour under chip antenna unless required by antenna datasheet

Maintain antenna orientation consistently between prototypes

Validate RSSI/SNR in final housing

9.4 RF vs. Mechanical Trade-Off

A metal-reinforced housing improves structural strength but can degrade RF performance. A polymer housing improves RF but may need reinforcement for road loads.

Recommended compromise:

Hybrid structure:

Metal or reinforced base for strength

Polymer upper or side RF window for LoRa antenna

Antenna located behind RF-transparent wall

10. Accelerometer Placement

10.1 Recommended Location

The accelerometer should be mounted rigidly near the lower structural base of the reflector.

Recommended placement:

Lower PCB or small sensor PCB

Rigidly connected to housing base

Near central structural region

Away from loose covers or flexible panels

Away from power regulator heat

Away from piezo cantilever resonance path

10.2 Why the Accelerometer Should Be Rigidly Mounted

The accelerometer should measure actual reflector or pavement-coupled vibration. If it is mounted on a flexible support, foam, or loose PCB, it may measure the internal resonance of the structure rather than the roadway vibration.

Correct structure:

Pavement

↓

Adhesive layer

↓

Rigid reflector base

↓

Rigid internal mount

↓

Accelerometer PCB

Avoid:

Accelerometer floating on wires

Accelerometer mounted with soft silicone

Accelerometer attached to flexible top cover

Accelerometer placed on same cantilever as piezo

Accelerometer near moving battery holder

10.3 Recommended Mounting Method

For the first prototype:

Mount accelerometer on main PCB if the PCB is rigidly attached to the base

or

Use a small dedicated sensor PCB bolted or bonded to the structural base

For better repeatability, the dedicated sensor PCB is preferred.

10.4 Placement Relative to Center of Housing

The accelerometer should initially be placed near the central lower region of the reflector. This provides a more balanced measurement and reduces sensitivity to local housing flex.

Recommended first placement:

Center-bottom of housing

Rigid base mount

Known X/Y/Z orientation

Orientation should be marked in CAD and firmware documentation.

10.5 Why It Should Be Separated From the Piezo Cantilever

The piezo cantilever is intentionally designed to amplify vibration. The accelerometer should not be placed on that same structure because it may capture exaggerated resonance rather than true road vibration.

Recommended separation:

Piezo:

Flexible amplified structure

Accelerometer:

Rigid reference structure

11. Piezoelectric Sensor Placement

11.1 Recommended Location

The piezoelectric sensor should be mounted inside the reflector as a mechanically amplified wake-up element.

Recommended placement:

Lower internal zone

Anchored to rigid base

Mounted as cantilever or flexing element

Separated from battery and antenna

Protected from direct impact

Not fully potted in rigid resin

11.2 Purpose

The piezoelectric element is not the primary precision measurement sensor. Its main function is to detect mechanical activity and wake the ESP32-C3.

Function:

Road vibration or impact

↓

Piezo mechanical deformation

↓

Electrical pulse

↓

Wake-up circuit

↓

ESP32-C3 deep sleep wake

11.3 Recommended Cantilever Structure

A cantilever structure is recommended for mechanical amplification.

Rigid anchor point

│

├──── flexible beam ─────● small mass

↑

piezo bonded near high-strain region

Recommended initial values for prototype testing:

Beam length: 30 mm to 60 mm

Beam width: 8 mm to 15 mm

Beam thickness: 0.3 mm to 1.0 mm

Tip mass: 1 g to 10 g

Initial mass target: 3 g to 5 g

The piezo should be bonded near the fixed end, where strain is higher.

11.4 Why the Piezo Should Not Be Fully Rigidly Mounted

If the piezo is rigidly clamped across its entire surface, it may not flex enough to generate a useful signal.

Good mounting:

One controlled fixed region

Flexible region allowed to bend

Tip mass for amplification

Mechanical stop to prevent over-flexing

Poor mounting:

Fully embedded in hard potting

Loose inside housing

Pressed directly by battery

Exposed to tire impact

Placed near antenna with metal mass

11.5 Overload Protection

The cantilever should include a mechanical stop to prevent the beam from bending too far during shock events.

Recommended feature:

Small internal stop above/below cantilever

Gap sized to allow normal vibration

Stop prevents destructive deflection

12. Temperature Sensor Placement

12.1 Battery NTC

The most important temperature sensor is the battery NTC.

Location:

Directly attached to LiFePO₄ 18650 cell body

Mid-cell side wall

Thermally coupled

Electrically insulated if needed

Purpose:

Disable charging below 0 °C

Disable charging above high limit

Log battery temperature

Protect battery lifetime

12.2 Internal Air Temperature Sensor

A second temperature sensor should measure internal enclosure temperature.

Location:

Upper-middle internal cavity

Away from direct contact with charger

Away from direct contact with battery

Away from direct solar panel conduction path

Purpose:

Estimate enclosure thermal environment

Detect solar heating

Support thermal model

Trigger reduced operation if enclosure overheats

12.3 Power Stage Temperature Sensor

A third temperature sensor is recommended near the charger and regulator.

Location:

Near CN3058E and buck-boost regulator

On PCB close to thermal copper

Not directly on the IC package

Purpose:

Detect charger overheating

Detect regulator overheating

Validate thermal design

Protect system during high solar input

12.4 Optional Solar Panel Temperature Sensor

For early prototypes, an optional sensor near the solar panel/top cover is useful.

Purpose:

Measure top surface heating

Correlate solar exposure with internal temperature

Improve enclosure thermal model

This sensor may be removed in final production if the data shows it is not needed.

13. Solar Panel Placement

13.1 Recommended Location

The solar panel should be placed on the upper surface of the reflector, protected by a transparent or translucent impact-resistant cover.

Recommended placement:

Top surface

Slightly recessed or flush protected

Mechanically isolated from direct tire shear when possible

Sealed around perimeter

Wired directly to power input protection

13.2 Design Considerations

The solar panel must balance energy harvesting with mechanical survival.

Risks:

- Tire abrasion

- Debris impact

- Snow and dirt coverage

- UV degradation

- Water ingress around edges

- Heat transfer into battery zone

- Reduced efficiency from road grime

Recommended protections:

Recessed panel pocket

UV-stabilized transparent cover

Sealed perimeter

Drainage consideration if applicable

Impact-resistant material above panel

Replaceable panel in prototype stage

13.3 Thermal Consideration

The solar panel can become a heat source under direct sun. The battery should not be directly attached to the solar panel back side.

Recommended separation:

Solar panel

↓

Air gap or thermal barrier

↓

Battery zone offset laterally or lower in structure

14. Protection Circuit Placement

14.1 Battery Protection

Protection circuitry should be close to the battery connection.

Required functions:

Overcharge protection

Overdischarge protection

Short-circuit protection

Reverse polarity protection

Current limiting

Fuse or PTC protection

Recommended location:

Near battery terminal path

Before system power distribution

Accessible to test points in prototype

Separated from antenna area

14.2 Solar Input Protection

Solar input protection should be placed close to the solar panel entry point.

Recommended components/functions:

Reverse current blocking

TVS or transient protection

Input capacitor

Current limiting if required

Connector protection

Purpose:

- Protect charger from transients

- Prevent reverse leakage into panel

- Reduce EMI

- Improve charging reliability

15. Load Switch for Accelerometer

15.1 Recommended Placement

The accelerometer load switch should be placed near the accelerometer power input.

Recommended connection:

3.3 V rail

↓

Load switch controlled by ESP32-C3

↓

VDD_ACCEL

↓

Accelerometer

15.2 Reasoning

The load switch allows the accelerometer to consume approximately zero current when the system is asleep.

This supports the ultra-low-power architecture:

Piezo stays active as wake-up source

Accelerometer remains fully unpowered

ESP32 wakes

ESP32 enables accelerometer

Accelerometer captures data

ESP32 disables accelerometer

15.3 Back-Powering Prevention

If the accelerometer is unpowered, communication lines must not accidentally power it through I²C or SPI pins.

Recommended precautions:

Place I²C pull-ups on switched VDD_ACCEL

Set ESP32 pins to high impedance before powering off sensor

Use series resistors if needed

Avoid always-on pull-ups to 3.3 V on sensor lines

Validate leakage current in deep sleep

16. PCB Placement Strategy

16.1 Main PCB

The main PCB should contain:

- ESP32-C3

- LoRa module interface

- CN3058E charger

- Buck-boost regulator

- Protection circuitry

- Battery voltage measurement

- Temperature sensing inputs

- Accelerometer interface

- Load switch control

- Debug/test points

Recommended PCB placement:

Middle-to-lower internal layer

Mechanically attached to internal frame

Thermally separated from top solar panel

Rigidly connected near accelerometer zone

With antenna keep-out region respected

16.2 Separate Sensor PCB Option

A separate accelerometer PCB may improve measurement repeatability.

Advantages:

- Better mechanical coupling

- Easier orientation control

- Less noise from power circuits

- Easier comparison of different accelerometers

- Better modularity during testing

Disadvantages:

- More connectors or solder points

- More assembly complexity

- Higher prototype cost

Recommended prototype approach:

Use separate accelerometer PCB for testing

Integrate onto main PCB only after mechanical behavior is validated

17. Sealing and Environmental Protection

17.1 Seal Location

The main seal should be located away from high-impact contact areas when possible.

Recommended sealing strategy:

Perimeter gasket or adhesive seal

Conformal coating on PCB

Selective potting for high-risk areas

No full potting of battery

No flexible potting directly around accelerometer

Controlled cable/solar feedthrough sealing

17.2 Condensation Risk

A sealed housing can trap moisture during assembly or field operation.

Mitigation options:

Dry assembly process

Desiccant in prototype units

Conformal coating

Hydrophobic vent for pressure equalization if suitable

Corrosion-resistant contacts

Avoid exposed copper

17.3 Road Salt Protection

Road salt creates corrosion risk.

Recommended protections:

Conformal coating

Sealed battery terminals

Gold-plated or protected contacts where needed

No exposed steel near electronics

Stainless or coated fasteners

Avoid galvanic metal pairs

18. Recommended Final Internal Layout

The recommended first complete prototype layout is:

Top:

Solar panel with protected cover

Upper side/corner:

LoRa antenna behind polymer wall

Middle protected zone:

LiFePO₄ 18650 battery with direct NTC

Main PCB:

CN3058E charger near solar/battery path

Buck-boost regulator near load distribution

ESP32-C3 near control and communication interfaces

LoRa module near antenna feed

Protection circuitry near power entry and battery

Lower rigid zone:

Accelerometer mounted to structural base

Piezoelectric cantilever mounted to rigid base

Thermal sensors:

NTC 1 on battery

NTC 2 internal enclosure air

NTC 3 near power electronics

Optional NTC 4 near solar panel/top cover

Simplified layout:

┌─────────────────────────────────────────────┐

│ Solar panel / protected top cover            │

├─────────────────────────────────────────────┤

│ RF Zone: antenna near polymer side wall      │

│ Internal temp sensor                         │

├─────────────────────────────────────────────┤

│ Battery Zone: LiFePO₄ 18650 + battery NTC    │

├─────────────────────────────────────────────┤

│ Main PCB: CN3058E / buck-boost / ESP32 / LoRa│

│ Power-stage temp sensor                      │

├─────────────────────────────────────────────┤

│ Sensor Zone: rigid accelerometer mount       │

│ Piezo cantilever + tip mass                  │

└─────────────────────────────────────────────┘

Pavement bonding surface

19. Component Placement Matrix

| Component | Recommended Location | Reason |
| --- | --- | --- |
| LiFePO₄ 18650 | Middle protected battery zone | Reduces direct solar and pavement thermal exposure |
| Battery NTC | Directly attached to battery cell | Required for safe charge inhibition |
| CN3058E | Power zone near battery and solar input | Minimizes power path length and manages charger heat |
| Buck-boost regulator | Main PCB near load distribution | Supports stable 3.3 V rail and current peaks |
| ESP32-C3 | Central electronics zone | Controls wake, sensing, power, and communication |
| LoRa module | Near antenna feed | Reduces RF feed losses |
| LoRa antenna | Side/upper RF-transparent region | Minimizes shielding by battery/metal |
| Accelerometer | Lower rigid structural base | Captures true reflector/pavement vibration |
| Piezoelectric sensor | Lower cantilever structure | Amplifies vibration for wake-up |
| Internal temp sensor | Upper-middle internal cavity | Measures enclosure thermal environment |
| Power temp sensor | Near charger/regulator | Detects local heat generation |
| Solar panel | Top protected surface | Maximizes energy harvesting |
| Solar protection | Near solar panel entry | Protects charger from transients |
| Battery protection | Near battery terminals | Protects against electrical faults |
| Debug pads | Accessible edge or bottom service area | Supports prototype testing |

20. Prototype Validation Plan

20.1 Mechanical Layout Validation

Tasks:

Create CAD model

Place real 18650 model

Place PCB outline

Place solar panel

Place antenna keep-out zone

Place piezo cantilever

Place accelerometer mount

Check assembly clearances

Check seal path

Check screw/fastener locations

Deliverables:

- Internal layout drawing

- Exploded view

- Clearance analysis

- Assembly sequence

20.2 Thermal Validation

Tasks:

Measure battery temperature

Measure enclosure air temperature

Measure charger/regulator temperature

Measure solar panel temperature

Test cold environment

Test hot enclosure condition

Validate charge disable below 0 °C

Validate high-temperature cutoff

Deliverables:

- Thermal map

- Charge inhibit validation

- Summer overheating risk report

- Battery safety recommendation

20.3 Sensor Validation

Tasks:

Compare accelerometer placement options

Validate rigid mounting

Test piezo cantilever response

Measure wake-up success rate

Measure false positives

Capture vehicle vibration events

Compare walking, tapping, and vehicle events

Deliverables:

- Sensor placement recommendation

- Piezo beam/mass recommendation

- Accelerometer threshold settings

- Event feature dataset

20.4 RF Validation

Tasks:

Measure LoRa RSSI/SNR with housing open

Measure LoRa RSSI/SNR with housing closed

Test antenna near different walls

Test antenna near battery vs. away from battery

Test orientation sensitivity

Test ground-level range

Deliverables:

- Antenna location recommendation

- Housing material RF assessment

- LoRa range report

20.5 Environmental Validation

Tasks:

Water ingress screening

Condensation screening

Salt exposure screening

Freeze-thaw cycling

Vibration exposure

Drop/impact testing

Adhesive bond testing

Deliverables:

- Environmental risk report

- Seal design recommendation

- Material recommendation

21. Project-Level Next Steps

21.1 Immediate Prototype Actions

The next prototype should include:

Smart/solar raised marker housing

LiFePO₄ 18650

CN3058E charger

3.3 V buck-boost regulator

ESP32-C3

LoRa module

BMA400 accelerometer or equivalent

Piezoelectric wake-up circuit

Three NTC sensors

Protected solar panel

Accessible debug/test points

21.2 Build Two Internal Layout Variants

Recommended variants:

Variant A:

Battery centered, antenna on side wall, accelerometer on lower center base

Variant B:

Battery offset to one side, antenna opposite side, accelerometer on separate lower sensor PCB

Purpose:

- Compare RF performance

- Compare thermal behavior

- Compare vibration signal quality

- Compare assembly practicality

21.3 Data Collection Priorities

The prototype must log:

Battery voltage

Battery temperature

Internal enclosure temperature

Power-stage temperature

Solar input state

Charge enable/disable state

Wake-up source

Piezo event amplitude

Accelerometer event features

LoRa RSSI/SNR

System current profile

21.4 Design Freeze Criteria

The internal placement should not be frozen until the following are validated:

Battery remains within safe temperature limits

Charging is blocked below 0 °C

Antenna performs acceptably inside housing

Accelerometer detects vehicle-induced vibration

Piezo reliably wakes system

False wake-ups are manageable

Solar charging works in sealed housing

No water ingress path compromises electronics

Assembly process is realistic

Cost is compatible with project targets

22. Final Recommendation

The recommended internal placement strategy is:

Battery:

Middle protected zone with direct NTC contact

CN3058E and regulator:

Main PCB power zone with thermal copper and local temperature monitoring

ESP32-C3:

Central control zone on main PCB

LoRa antenna:

Upper/side RF-transparent zone away from battery, metal, and copper

Accelerometer:

Lower rigid base, mechanically coupled to reflector structure

Piezoelectric sensor:

Lower internal cantilever with tip mass for wake-up amplification

Solar panel:

Top protected surface with thermal separation from battery

Temperature sensors:

Battery cell, internal enclosure, and power electronics zone

The first complete prototype should prioritize measurement access, internal volume, and layout flexibility. The layout should not be aggressively miniaturized until the thermal, RF, and vibration behavior are measured.

The most important design rule is:

Rigid accelerometer, flexible piezo, thermally monitored battery, isolated RF antenna, and controlled charger heat.

This placement strategy gives the best balance between sensor performance, battery safety, low-power operation, RF reliability, and future product scalability.