Technical Analysis: Thermal Management & Structural Housing for Smart Road Infrastructure

Document Reference: TA-2026-003
Project: SmartReflect Platform Integration
Prepared For: Smart Road Reflector Prototype Development

1. Problem Statement

Autonomous, long-term deployment of smart pavement intelligence nodes requires a highly optimized physical housing. The system must operate reliably in harsh outdoor roadway environments while maintaining structural integrity, sensor performance, RF communication reliability, and long-term energy autonomy.

The primary engineering challenge is to identify the best road reflector housing architecture that balances the following competing requirements:

- Cost

- Physical size

- Internal volume

- Mechanical strength

- Material durability

- RF signal transmission

- Solar exposure

- Thermal protection

- Battery safety

- Cold-weather operation

- Summer heat rejection

- Water and salt resistance

- Snowplow compatibility

- Installation complexity

- Manufacturability

- Long-term serviceability

The housing must support a complete embedded system, including:

- LiFePO₄ 18650 battery

- CN3058E LiFePO₄ charger

- Buck-boost regulator

- ESP32-C3 microcontroller

- LoRa radio module

- Piezoelectric wake-up sensor

- Accelerometer

- NTC temperature sensors

- Solar input or external energy input

- Sealed enclosure

- Mechanical coupling to pavement vibration

The problem is not only to fit electronics inside a road reflector. The real problem is to design a road-rated enclosure that can protect the electronics, transfer useful vibration to the sensors, survive Ohio roadway conditions, and remain economically scalable.

2. Key Engineering Problem

The project must find the optimal balance between structural ruggedness, thermal control, sensor performance, RF performance, and cost.

The most important trade-offs are:

Low profile vs. internal volume

Mechanical strength vs. RF transparency

Thermal insulation vs. summer heat rejection

Battery protection vs. solar charging efficiency

Standard road-marker geometry vs. custom smart-node integration

Low cost vs. long-term field reliability

Prototype flexibility vs. production manufacturability

For Ohio road deployment, the design must handle:

- Cold winters

- Snow and ice

- Snowplow interaction

- Road salt

- Freeze-thaw cycles

- Rain and standing water

- Summer pavement heating

- Vehicle impact and tire loading

- Vibration and shock

- UV exposure

- Limited winter solar energy

The housing must be treated as a functional subsystem, not just a plastic shell.

3. Functional Requirements

The reflector housing must provide the following functions:

3.1 Mechanical Protection

The enclosure must protect the internal electronics from:

- Tire loads

- Impact

- Vibration

- Installation stress

- Pavement movement

- Snowplow contact

- Debris impact

- Compression loads

- Repeated thermal expansion and contraction

3.2 Environmental Protection

The enclosure must protect against:

- Water ingress

- Condensation

- Road salt

- Oil and road contaminants

- Dust

- Ice formation

- UV degradation

- Thermal cycling

Target environmental protection should be:

Minimum target: IP67

Preferred target: IP68-like sealed design for final product

3.3 Thermal Management

The enclosure must prevent the battery and electronics from operating outside safe limits.

Main thermal requirements:

Prevent LiFePO₄ charging below 0 °C

Prevent charging above approximately 45 °C

Reduce high-power operation above approximately 60 °C

Protect electronics during internal enclosure temperatures near 70 °C

Avoid trapping excessive heat during summer

Avoid allowing battery temperature to drop too rapidly during winter

3.4 RF Transmission

The enclosure must allow LoRa communication.

This creates material constraints:

Metal enclosure:

Excellent mechanical strength

Poor RF transmission unless antenna is externally exposed or isolated

Plastic/polymer enclosure:

Good RF transmission

Lower structural strength unless reinforced

Hybrid enclosure:

Best balance if metal is used as structural base and polymer window is used for RF path

3.5 Sensor Coupling

The enclosure must transfer useful vibration to the sensors.

The accelerometer should be mounted rigidly to a structural reference point. The piezoelectric sensor may use a cantilever or amplified mechanical structure.

Accelerometer:

Rigid mechanical coupling preferred

Piezoelectric sensor:

Cantilever/mass structure preferred for mechanical amplification

4. System Architecture Assumption

The housing will support the following low-power operating concept:

System in deep sleep

↓

Piezoelectric sensor detects vibration

↓

Wake-up circuit triggers ESP32-C3

↓

ESP32-C3 enables accelerometer power

↓

Accelerometer captures a short vibration window

↓

ESP32-C3 processes the event

↓

LoRa sends a compact message

↓

System returns to deep sleep

This strategy minimizes sensor and radio energy consumption while allowing the system to detect vehicle events.

5. Candidate Reflector Housing Options

Four road-reflector housing concepts are evaluated:

- Low-profile square/trapezoidal raised pavement marker

- Round button raised pavement marker

- Smart/solar raised pavement marker

- Snowplowable recessed pavement marker

6. Option A: Low-Profile Square / Trapezoidal Raised Marker

6.1 Description

This form factor resembles common non-plowable raised pavement markers. A typical reference size is approximately:

Length: ~100 mm

Width: ~100 mm

Height: ~18–20 mm

This option provides the most compact and standardized geometry.

6.2 Advantages

- Lowest profile

- Lower material usage

- Lower cost potential

- Good aerodynamic/tire interaction

- Easier to visually integrate with existing road markers

- Lower protrusion above pavement

- Less likely to be directly impacted than taller housings

- Simple geometry for early mechanical prototyping

6.3 Disadvantages

- Very limited internal height

- Difficult to fit an 18650 cell, PCB, antenna, insulation, and sensor structure

- Limited room for thermal separation

- Limited room for piezoelectric mechanical amplification

- Limited room for battery retention and shock isolation

- Difficult to include a solar panel with useful surface area

- Poor fit for field-serviceable design

- May be unsuitable for roads with snowplow exposure unless specifically designed as plowable or recessed

6.4 Thermal Assessment

The low internal volume makes thermal control difficult.

Main thermal concerns:

Small air volume heats quickly in summer

Limited insulation volume for winter

Battery is close to outer shell

Electronics may be thermally coupled to pavement temperature

CN3058E heat cannot be controlled effectively

The CN3058E linear charger may generate localized heat during charging, but using charger inefficiency as a deliberate heater is not recommended as the primary thermal strategy. It is unpredictable, depends on solar input, and may create summer overheating risk.

6.5 Integration Assessment

This option is mechanically attractive but electrically restrictive.

Fitting an 18650 cell is possible only with aggressive internal packaging. The 18 mm battery diameter nearly consumes the full internal height of a 20 mm marker, leaving little room for protective wall thickness, adhesive, PCB clearance, and thermal insulation.

6.6 Recommendation

This option is acceptable for:

Non-solar prototype

Small electronics-only version

Short-term bench/field test

External battery test setup

It is not the best first housing for a complete solar-powered smart reflector using a LiFePO₄ 18650 cell.

Recommended status:

Early proof-of-concept: Possible

Full smart node prototype: Not preferred

Final product: Only if electronics are miniaturized significantly

7. Option B: Round Button Raised Marker

7.1 Description

This form factor uses a circular or dome-like housing.

Reference geometry:

Diameter: ~120–130 mm

Height: ~25 mm

The round shape provides more usable horizontal space than the square low-profile marker while maintaining a compact footprint.

7.2 Advantages

- Better internal volume than low-profile square marker

- Circular layout supports centralized PCB design

- More room for battery placement

- More room for antenna separation

- Better opportunity for internal thermal zoning

- Better opportunity for piezoelectric cantilever structure

- Good candidate for prototype iteration

- Can provide more solar panel surface area than a small square marker

7.3 Disadvantages

- Taller than low-profile markers

- More exposed to tire impact

- Higher impact stress at highway speed

- May be more vulnerable to snowplows

- Requires stronger material selection

- Requires careful sloped/domed geometry to reduce impact concentration

- More custom than standard square markers

- Potentially higher tooling cost

7.4 Thermal Assessment

The round button format gives more freedom for thermal design.

Possible thermal zones:

Central warm zone:

Battery + NTC + charger

Outer electronics zone:

MCU + LoRa + sensors

Upper solar zone:

Solar panel + transparent or translucent cover

Lower structural zone:

Rigid base coupled to pavement vibration

The increased volume allows limited use of insulation, air gaps, and internal thermal barriers. However, a sealed round marker may still overheat in summer if it has a dark top surface or poor heat rejection.

7.5 RF Assessment

A polymer round housing is favorable for LoRa transmission. If metal reinforcement is used, the antenna should be located near a polymer window or isolated from the metal structure.

7.6 Integration Assessment

This is a strong prototype option because it provides enough space to test different internal layouts without immediately requiring extreme miniaturization.

7.7 Recommendation

The round button marker is a good intermediate option for early integrated prototypes.

Recommended status:

Early integrated prototype: Recommended

Field prototype: Possible with ruggedization

Final product: Possible if snowplow risk is low or installation location is protected

8. Option C: Smart / Solar Raised Marker

8.1 Description

This form factor is designed natively for electronics and solar charging.

Reference geometry:

Length: ~125 mm

Width: ~125 mm

Height: ~25 mm

This option resembles commercial solar road studs or smart raised pavement markers.

8.2 Advantages

- Designed for embedded electronics

- More internal volume

- Better battery integration

- Better solar panel integration

- Better thermal zoning

- Better room for antenna placement

- Better room for sealing features

- Better room for mechanical sensor amplification

- Best fit for a complete functional prototype

- Easier to integrate CN3058E, battery, buck-boost regulator, LoRa, and sensors

8.3 Disadvantages

- Higher unit cost

- Larger footprint

- More visible as a custom device

- Higher installation adhesive area

- Higher exposure to tire impact

- May not be compatible with snowplow-heavy road sections unless specifically engineered

- Requires more careful material selection

- May require custom tooling for production

8.4 Thermal Assessment

This is the strongest thermal-management option among raised non-recessed housings.

Possible thermal strategy:

Battery located in an insulated cavity

NTC mounted directly to battery

CN3058E placed near but thermally controlled relative to battery

Buck-boost regulator separated from battery

Solar panel thermally decoupled from battery where possible

Internal air gap used as thermal buffer

Light-colored or reflective top surface used where possible

Firmware blocks charging below 0 °C and above high-temperature limit

The design should not rely on charger heat as the primary winter-heating method. Charger heat may be treated as incidental heat, but battery safety must be controlled by temperature-based charge enable/disable logic.

8.5 RF Assessment

A polymer top and side structure allows better RF behavior. The LoRa antenna should be placed away from the battery, copper pours, metal screws, and any metal reinforcement.

Recommended RF strategy:

Keep antenna near polymer wall

Avoid metal directly above or beside antenna

Maintain ground clearance according to antenna datasheet

Avoid fully metallic enclosures

Test RSSI/SNR with final housing material

8.6 Integration Assessment

This is the best option for the first complete field-capable smart-node prototype because it provides enough space for electronics, energy storage, thermal testing, and sensor optimization.

8.7 Recommendation

Recommended status:

Complete prototype: Strongly recommended

Field prototype: Recommended

Final product: Strong candidate if cost is acceptable

9. Option D: Snowplowable Recessed Pavement Marker

9.1 Description

A snowplowable or recessed pavement marker is designed so that snowplows can pass over it with reduced direct impact. Traditional versions may use a cast or reinforced housing, with the reflective or illuminated element protected inside a recessed or ramped structure.

For Ohio roads, this option deserves serious consideration because snowplow interaction is a major deployment risk.

9.2 Advantages

- Best snowplow survivability

- Better protection from direct blade impact

- More suitable for public road deployment in snow regions

- Better long-term mechanical durability

- Can protect optical and electronic elements

- More realistic for Ohio roadway conditions

- Lower risk of catastrophic housing removal

9.3 Disadvantages

- Highest installation complexity

- May require pavement slot cutting or recess preparation

- More expensive installation process

- More difficult to prototype quickly

- Metal housings may block LoRa RF transmission

- Limited internal geometry depending on marker style

- More difficult battery replacement

- More difficult waterproof sealing

- May need external or isolated antenna region

- May require specialized road contractor support

9.4 Thermal Assessment

A recessed design is thermally coupled to the pavement more strongly than raised polymer markers.

This can be beneficial in some cases:

Winter:

Pavement thermal mass may reduce rapid temperature swings

Summer:

Pavement and asphalt may transfer significant heat into the housing

Thermal behavior must be measured, not assumed.

9.5 RF Assessment

If the housing uses metal or cast structure, RF performance can be a major concern. The antenna may need:

- RF-transparent window

- External protected antenna zone

- Polymer insert

- Carefully placed antenna cavity

- Ground plane tuning

- Field RSSI/SNR validation

9.6 Integration Assessment

This option is not the fastest prototype path, but it may be the most realistic final product path for Ohio public roads.

9.7 Recommendation

Recommended status:

Early prototype: Not recommended

Controlled field prototype: Possible

Final Ohio road product: Strong candidate

10. Material Options

10.1 ABS

Advantages

- Low cost

- Easy to mold

- Good impact resistance

- Common in road marker products

- Good for prototypes

Disadvantages

- UV degradation unless stabilized

- Lower high-temperature performance than engineering plastics

- May deform under sustained heat and load

- Requires additives or coatings for outdoor durability

Recommended use:

Early prototype housing

Non-final low-cost enclosure

10.2 Polycarbonate

Advantages

- High impact resistance

- Better toughness than standard ABS

- Good RF transparency

- Good candidate for top cover

- Available in UV-stabilized grades

- Can be transparent or translucent for solar/light functions

Disadvantages

- Higher cost than ABS

- Can scratch

- Chemical resistance must be verified

- Needs UV-stabilized grade for outdoor use

Recommended use:

Top cover

RF-transparent region

Solar window

Impact-resistant shell

10.3 PMMA / Acrylic

Advantages

- Good optical clarity

- Good UV resistance

- Useful for lenses or solar windows

- Good surface hardness

Disadvantages

- More brittle than polycarbonate

- Lower impact resistance

- Not ideal as main structural body

Recommended use:

Lens area

Optical window

Protected solar cover

10.4 Aluminum

Advantages

- Strong

- Good thermal conductivity

- Good heat spreading

- Good mechanical base material

- Corrosion-resistant if treated properly

Disadvantages

- Blocks or detunes RF signals

- Conducts heat into and out of battery region

- Can create galvanic corrosion concerns

- Higher manufacturing cost

- May require polymer RF window

Recommended use:

Structural base

Heat spreader

Reinforcement insert

Not recommended as full RF enclosure

10.5 Stainless Steel

Advantages

- Excellent durability

- Excellent corrosion resistance

- Strong for snowplow and impact protection

Disadvantages

- High cost

- Difficult manufacturing

- Heavy

- Poor RF transparency

- Strong thermal conduction

- May be excessive for early prototype

Recommended use:

Snowplow protection rails

Fasteners

Localized reinforcement

Not recommended as full smart-node enclosure

10.6 Potting Compounds

Advantages

- Waterproofing

- Vibration protection

- Mechanical stabilization

- Tamper resistance

Disadvantages

- Can trap heat

- Makes rework difficult

- Can stress components during thermal cycling

- Can reduce sensor sensitivity if applied incorrectly

- Can make battery replacement impossible

Recommended use:

Selective potting for electronics

Avoid fully potting battery

Avoid potting accelerometer in flexible material if vibration accuracy is needed

11. Thermal Management Strategy

11.1 Key Thermal Risks

The main thermal risks are:

LiFePO₄ charging below 0 °C

Battery overheating during summer

Internal enclosure heating above ambient

CN3058E heat concentration

Buck-boost regulator heat

Solar panel heating

Poor heat dissipation in sealed enclosure

Sensor drift due to temperature

Condensation and freeze-thaw stress

11.2 Required Thermal Sensors

Minimum recommended sensor placement:

NTC 1:

Directly attached to battery cell

NTC 2:

Internal air/enclosure temperature

Optional NTC 3:

Near charger/regulator thermal zone

Optional NTC 4:

Near solar panel or top cover

11.3 Firmware Thermal Control

Minimum LiFePO₄ thermal logic:

If battery temperature < 0 °C:

Disable solar charging

If battery temperature is between 0 °C and 45 °C:

Allow charging

If battery temperature > 45 °C:

Disable or reduce charging

If internal temperature > 60 °C:

Disable high-power functions

If internal temperature > 70 °C:

Enter protective shutdown mode

11.4 CN3058E Heat Consideration

The CN3058E is a linear charger. It dissipates heat based on input voltage, battery voltage, and charge current.

Approximate thermal loss:

Power loss ≈ (VIN - VBAT) × ICHG

Example:

VIN = 5.0 V

VBAT = 3.2 V

ICHG = 0.2 A

Power loss ≈ (5.0 - 3.2) × 0.2

Power loss ≈ 0.36 W

This heat can warm the PCB locally, but it should not be treated as a reliable battery heater.

Risks of using charger heat as a heater:

- Heat only exists when solar input is available

- Heat depends on battery state of charge

- Heat increases summer thermal risk

- Heat may be localized at charger IC, not battery core

- Charge should be disabled below 0 °C for LiFePO₄ anyway

- Using charge current to heat a cold LiFePO₄ cell can be unsafe

Preferred strategy:

Use temperature-based charge blocking

Use insulation to slow battery cooling

Use low-power operation during freezing conditions

Evaluate LTO chemistry if cold charging becomes mandatory

12. Structural Design Strategy

12.1 Internal Zoning

The enclosure should be divided into functional zones:

Zone A: Battery thermal zone

- LiFePO₄ 18650

- NTC attached to battery

- Mechanical retention

- Thermal insulation

Zone B: Power electronics zone

- CN3058E

- Buck-boost regulator

- Protection circuitry

- Solar input protection

Zone C: Sensor zone

- Accelerometer rigidly mounted

- Piezoelectric cantilever or mechanical amplifier

Zone D: RF zone

- LoRa antenna

- RF-transparent housing wall

- Separation from metal and battery

Zone E: Optical/solar zone

- Solar panel

- Transparent or translucent protective cover

12.2 Battery Mounting

The battery must be retained mechanically.

Required features:

- No loose cell movement

- Shock absorption without excessive thermal insulation

- No direct compression on cell casing

- Electrical insulation

- Corrosion-resistant contacts

- Strain relief on wires

- Controlled venting or safe failure path

- NTC bonded to the cell

12.3 PCB Mounting

The PCB should be mounted to a rigid internal frame.

Requirements:

- Mechanical support against vibration

- No PCB flex near accelerometer

- Thermal isolation from high-heat components

- Coating or sealing against moisture

- Test points available in prototype stage

- Connector strategy for debugging

12.4 Sensor Mounting

Recommended approach:

Accelerometer:

Rigid mount to structural base

Piezoelectric sensor:

Cantilever with small mass for wake-up signal amplification

The accelerometer should not be mounted on a flexible cantilever in the first prototype, because it may measure the resonance of the cantilever rather than true pavement vibration.

13. Comparative Housing Matrix

| Parameter | Option A: Low-Profile Square | Option B: Round Button | Option C: Smart/Solar Marker | Option D: Snowplowable Recessed |
| --- | --- | --- | --- | --- |
| Internal volume | Low | Medium | High | Medium |
| 18650 integration | Difficult | Possible | Good | Depends on structure |
| Solar integration | Poor/limited | Medium | Good | Difficult |
| Thermal zoning | Poor | Medium | Good | Medium |
| Snowplow survivability | Poor unless specially designed | Poor/medium | Poor/medium | High |
| RF performance | Good if polymer | Good if polymer | Good if polymer | Difficult if metal |
| Prototype speed | High | High | Medium | Low |
| Installation complexity | Low | Low/medium | Medium | High |
| Unit cost | Low | Medium | Medium/high | High |
| Product realism for Ohio | Low/medium | Medium | Medium | High |
| Best use | Miniaturized node | Early integrated prototype | Complete smart prototype | Final road-ready direction |

14. Preliminary Recommendation

The recommended housing development path is:

First complete prototype:

Option C - Smart/Solar Raised Marker

Secondary prototype:

Option B - Round Button Marker

Miniaturized future version:

Option A - Low-Profile Square Marker

Ohio road-ready final direction:

Option D - Snowplowable Recessed Marker

The first complete prototype should prioritize internal volume, thermal measurement, battery safety, and debugging flexibility. For this reason, the smart/solar raised marker form factor is the strongest starting point.

The final product for Ohio public roads should eventually evaluate a snowplowable or recessed architecture, because snowplow interaction is one of the most important long-term mechanical risks.

15. Prototype Development Plan

15.1 Phase 1: Bench Housing Prototype

Build a non-road-rated prototype enclosure to validate internal layout.

Goals:

- Confirm 18650 fit

- Confirm PCB fit

- Confirm antenna placement

- Confirm solar input routing

- Confirm thermal sensor placement

- Confirm piezoelectric sensor mounting

- Confirm accelerometer rigid mount

- Confirm service/debug access

Recommended prototype method:

3D printed enclosure

Clear top cover for inspection

Replaceable internal inserts

External test connectors

Removable battery

Deliverables:

- CAD model

- Internal component layout

- Wire routing plan

- Initial thermal zoning concept

- Mechanical sensor mounting plan

15.2 Phase 2: Thermal Prototype

Build a sealed or semi-sealed enclosure to test thermal behavior.

Test conditions:

Cold test:

-20 °C to -30 °C

Room temperature:

25 °C

Hot enclosure test:

60 °C to 70 °C

Solar exposure test:

Direct sun simulation

Thermal cycling:

Repeated cold/hot cycles

Data to log:

- Battery temperature

- Internal enclosure temperature

- Charger temperature

- Regulator temperature

- Solar panel temperature

- Battery voltage

- Charge enable/disable state

- System current

- LoRa event current

Deliverables:

- Thermal profile report

- Charge blocking validation

- Summer overheating risk assessment

- Winter operation assessment

15.3 Phase 3: Mechanical Vibration Prototype

Build a mechanically coupled prototype for vibration and wake-up testing.

Test goals:

- Measure pavement vibration transfer

- Compare piezo wake-up sensitivity

- Compare accelerometer signal quality

- Evaluate false positives

- Evaluate missed events

- Validate rigid accelerometer mounting

- Validate piezo cantilever design

Test events:

Walking near device

Manual tapping

Vehicle passing beside device

Vehicle passing over nearby pavement

Small car

Pickup truck

Heavy vehicle if available

Rain/wind false trigger test

Data to collect:

- Piezo wake pulse amplitude

- Wake-up success rate

- Accelerometer X/Y/Z

- Acceleration magnitude

- Dynamic magnitude

- Peak acceleration

- RMS vibration

- Event duration

- Dominant frequency estimate

Deliverables:

- Sensor mounting recommendation

- Wake-up threshold recommendation

- Mechanical amplification recommendation

- Event-detection data set

15.4 Phase 4: Outdoor Controlled Field Prototype

Install units in a controlled non-public or low-risk test area.

Goals:

- Validate enclosure sealing

- Validate solar charging

- Validate battery temperature behavior

- Validate LoRa communication

- Validate event detection

- Validate false-positive behavior

- Validate condensation resistance

- Validate mechanical durability

Recommended test duration:

Minimum: 2 weeks

Preferred: 1 to 3 months

Data to log:

- Battery voltage

- Battery temperature

- Internal temperature

- Solar input

- Charge state

- Wake-up events

- LoRa messages

- RSSI/SNR

- Accelerometer features

- Piezo events

- Environmental notes

16. Project-Level Next Steps

16.1 Define Housing Requirements

Before final design, define:

Maximum allowable footprint

Maximum allowable height

Expected installation location

Snowplow exposure level

Minimum required autonomy

Solar panel area

Required battery size

Expected service life

Target unit cost

Target installation method

Required ingress protection

Required mechanical load rating

RF range requirement

Maintenance/replacement strategy

16.2 Select Prototype Housing

Recommended first housing:

Smart/Solar Raised Marker form factor

Approximate size: 125 mm × 125 mm × 25 mm

Material: UV-stabilized polycarbonate or hybrid PC/ABS

Battery: LiFePO₄ 18650

Charger: CN3058E

Regulator: 3.3 V buck-boost

Sensor wake-up: piezoelectric

Accelerometer: BMA400 or selected candidate

16.3 Create CAD and Internal Layout

CAD model must include:

- Battery compartment

- PCB mounts

- Antenna keep-out

- Solar panel area

- NTC locations

- Piezo cantilever

- Accelerometer rigid mount

- Seal/gasket region

- Adhesive footprint

- Wire routing

- Debug connector access

- Drain/vent strategy if applicable

16.4 Build Housing Variants

Build at least three mechanical variants:

Variant 1:

Large smart/solar marker for full electronics integration

Variant 2:

Round button marker for compact integration

Variant 3:

Low-profile square marker for miniaturization assessment

Optional future variant:

Variant 4:

Snowplowable/recessed marker concept

16.5 Run Environmental and Structural Tests

Minimum test plan:

- Water ingress test

- Thermal cycling

- Cold exposure

- Hot exposure

- Solar heating test

- Salt exposure screening

- Vibration test

- Drop/impact test

- Tire load simulation

- Adhesive bonding test

- RF range test

- Battery charge inhibit test

16.6 Prepare Productization Review

Before moving to product design:

- Confirm final housing geometry

- Confirm battery thermal strategy

- Confirm RF antenna performance

- Confirm snowplow risk classification

- Confirm material selection

- Confirm installation method

- Confirm cost target

- Confirm supplier availability

- Confirm manufacturability

- Confirm standards and regulatory pathway

- Confirm field test results

17. Risk Register

| Risk | Severity | Likelihood | Mitigation |
| --- | --- | --- | --- |
| Battery charging below 0 °C | High | High in Ohio | NTC-based charge inhibit |
| Summer overheating | High | Medium/high | Thermal testing, light top surface, charge cutoff |
| Snowplow damage | High | High on public roads | Evaluate recessed/snowplowable design |
| RF attenuation | Medium/high | Medium | RF-transparent window, antenna testing |
| Water ingress | High | Medium | Gasket, potting, IP testing |
| Road salt corrosion | High | Medium | Sealed enclosure, coating, corrosion-resistant contacts |
| Low vibration transfer | Medium | Medium | Rigid sensor mount, piezo cantilever |
| Excess false wake-ups | Medium | Medium | Threshold tuning, dual-sensor validation |
| Limited internal volume | High | High | Use larger prototype housing first |
| High unit cost | Medium/high | Medium | Design-to-cost review after prototype |
| Solar panel damage | Medium/high | Medium | Recessed/covered solar window |
| Battery replacement difficulty | Medium | Medium | Define service strategy early |

18. Final Recommendation

The recommended development approach is to avoid starting with the smallest possible road reflector. A very low-profile marker is attractive for cost and appearance, but it creates severe integration and thermal constraints.

The first complete prototype should use a smart/solar raised marker form factor because it provides enough internal space for:

- LiFePO₄ 18650 battery

- CN3058E charger

- Buck-boost regulator

- ESP32-C3

- LoRa module

- Antenna

- Piezoelectric wake-up structure

- Accelerometer rigid mount

- NTC thermal monitoring

- Solar input

- Debugging and test access

Recommended prototype path:

Prototype 1:

Smart/solar raised marker housing

Prototype 2:

Round button compact housing

Prototype 3:

Low-profile square miniaturized housing

Future Ohio road-ready version:

Snowplowable recessed or protected marker architecture

The final Ohio product should strongly consider a snowplowable or recessed design. However, that design should not be the first prototype because it adds installation complexity, cost, RF challenges, and mechanical design constraints.

The immediate next step is to build a large-enough smart/solar housing prototype, measure real thermal behavior, validate battery charge protection, and confirm that the sensor system can detect roadway vibration reliably before reducing the enclosure size.